NOTE TO SPECIFYING ARCHITECT REGARDING

SECTION 11082

ELECTRIC PROJECTION SCREEN

This specification is frequently needed for electric screens on platforms.

Specify a square screen to allow the screen box to be hidden behind teaser

border.

For more information or clarification call 1-800-548-8982; no charge, of course.

SECTION 11082

ELECTRIC PROJECTION SCREEN

Picture screen:

Furnish and install one Da-Lite Senior Electrol picture screen, or approved equal, with heavy duty windings, ball bearings, and limit switches preset at the factory.  Screen size shall be 16 feet horizontal by 16 feet vertical.  Provide steel hangers, pipe clamps, welded chain, beam clamps and turnbuckles for leveling.  Provide 1 ½ inch (inside diameter) standard steel supporting pipe batten.  This contractor shall furnish and install the screen and shall deliver the control station to the electrical contractor on the job.   Connective wiring and mounting of the control station shall be performed by the electrical contractor, but the physical installation of the screen shall be made by the stage equipment contractor.  The one 3-button control station shall be located on the stage end wall 60  inches above finished floor in direct line with the screen being operated, or as directed by architect.

END OF SECTION

11082-1

11/09/04